# R8 Ego centric camera specification

1080P M12*P0 5 SC235HGS

Fixed  Focus  USB 2.0  Camera Module

This  R8 2MP  dual-lens  stereo  camera  module  is  purpose-built  for  egocentric  data  collection,  robot learning, and embodied AI research. Featuring dual global shutter sensors, it outputs synchronized stereo video at 30fps — eliminating rolling shutter distortion for reliable stereo matching and depth estimation. Hardware-triggered synchronization ensures pixel-perfect frame alignment between left and  right  cameras.  An  integrated  6-axis  IMU  delivers  configurable  sampling  at  100/200/500Hz, hardware-timestamped and synchronized with camera frames at nanosecond level for drift-free visual -inertial fusion. Each unit includes a unique factory calibration file with intrinsics, distortion coefficients, and extrinsics — eliminating onsite setup.  Every frame and  IMU sample shares  unified hardware timestamps with  complete  SDK/API support.  Plug-and-play  USB  2.0  with  cross-platform  SDK for Windows, Linux, and ROS enables rapid deployment.

R8 Key  Features:

l Global Shutter ——All pixels expose simultaneously, eliminating rolling shutter distortion and  motion artifacts during fast hand/body movement. Ensures geometrically intact stereo frames for reliable depth estimation and VIO/SLAM front-end processing.

l Built-in 6-Axis IMU – Simplified VIO Integration

Integrated accelerometer and gyroscope eliminates external inertial sensors. Provides motion- compensation data essential for VIO and SLAM pipelines without additional system complexity.

l Factory Calibration – Plug and Play, No Onsite Setup

Unique calibration file per unit — intrinsics, distortion coefficients, and stereo extrinsics.

Eliminates checkerboard-based calibration, reduces deployment time from hours to minutes.

l Unified Hardware Timestamp – Dataset-Ready Precision

Every camera frame and every IMU sample shares the same hardware clock, achieving

nanosecond-level alignment. Meets the precision standards required for professional egocentric benchmarks and VIO pipelines. Complete SDK/API provided for seamless integration.

1080P M12*0.5P SC235HGS Fixed Focus USB 2.0  Camera  Module

| Model: | R8 |
| --- | --- |
| Resolution | 1600*1200 |
| Frame Rate | 30 fps |
| Sensor Size | 1/2.6" |
| Pixel Size | 3.0μm*3.0μm |
| Dynamic Range | Linear mode: 61.56 dB / HDR mode: 91.56 dB |
| FOV | D= 180°/H=115°/V=80° |
| Lens TTL | 15.15±0.3mm |
| Focus Type | Fixed Focus |
| Interface | USB 2.0 |
| Output Format | H.264/H.265 |
| Auto Control | Saturation, Contrast, Acutance White Balance, Exposure |
| Function Description | ranging / Dual-camera calibration / 3D reconstruction / VR vision / Depth detection Dual-camera synchronization / Dual-camera |
| Voltage | DC 5V |
| Working Current | MAX 320mA |
| PCB Size | 100*94*60mm |
| Hardware Synchronization | All cameras synchronized to a common hardware trigger/shutter signal |
| IMU | Built-in 6-axis IMU in Head and Wrist units where required |
| IMU Sampling data | 100/200/500HZ |
| Timestamp | Unified timestamp for all camera frames and IMU |
| SDK/API | SDK/API to be provided |
| Calibration Data | Camera intrinsic parameters + distortion coefficients + extrinsic parameters where applicable |
| Working Temperature | 0°C to +60°C |
| Storage Temperature | -20°C to +70°C |
| Mount | M12*P0.5mm |

Compatibility with Windows, Android, Mac OS, Linux, or Raspberry Pi

Top View

Side View

Application Built for Egocentric Data Collection

R8 Head-Mounted Egocentric Capture — Record What the Human Sees

From kitchens to factories, lab to offices  — Ego Camera R8 is designed for large-scale deployment.

Built for human demonstration capture, embodied AI, and VIO/SLAM. Head-Mounted

◆ Global Shutter ◆ Hardware Sync ◆ Factory Pre-Calibrated

| FORMAT | RESOLUTION | FRAME RATE |
| --- | --- | --- |
|  |  | USB 2.0 |
| MJPG | 1920 x 1080 (2MP) | 30 FPS |
|  | 1280 x 720 (720 HD) | 30 FPS |
|  | 640 x 480 (VGA) | 30 FPS |
| YUY2 | 1920 x 1080 (1080P) | 15 FPS |
|  | 1280 x 720 (720) | 30 FPS |
|  | 640 x 480 (VGA) | 30 FPS |

Lens Specifications

Dimension

Camera  Reliability Test

| Reliability Inspection Item |  |  | Testing Method | Acceptance Criteria |
| --- | --- | --- | --- | --- |
| Category |  | Item |  |  |
| Environmental | Storage Temperature | High 60°C 96 Hours | Temperature Chamber | No Abnormal Situation |
|  |  | Low -20°C 96 Hours | Temperature Chamber | No Abnormal Situation |
|  | Operation Temperature | High 60°C 24 Hours | Temperature Chamber | No Abnormal Situation |
|  |  | Low -20°C 24 Hours | Temperature Chamber | No Abnormal Situation |
|  | Humidity | 60°C 80% 24 Hours | Temperature Chamber | No Abnormal Situation |
|  | Thermal Shock | High 60°C 0.5 Hours Low -20 °C 0.5 Hours Cycling in 24 Hours | Temperature Chamber | No Abnormal Situation |
| Physical | Drop Test (Free Falling) | Without Package 60 cm | 10 Times on Wood Floor | Electrically Functional |
|  |  | With Package 60cm | 10 Times on Wood Floor | Electrically Functional |
|  | Vibration Test | 50Hz X-Axis 2mm 30min | Vibration Table | Electrically Functional |
|  |  | 50Hz Y-Axis 2mm 30min | Vibration Table | Electrically Functional |
|  |  | 50Hz Z-Axis 2mm 30min | Vibration Table | Electrically Functional |
|  | Cable Tensile Strength Test | Loading Weight 4 kg 60 Seconds Cycling in 24 Hours | Tensile Testing Machine | Electrically Functional |
| Electrical | ESD Test | Contact Discharge 2 KV | ESD Testing Machine | Electrically Functional |
|  |  | Air Discharge 4 KV | ESD Testing Machine | Electrically Functional |
|  | Aging Test | On/ Off 30 Seconds Cycling in 24 Hours | Power Switch | Electrically Functional |
|  | USB Connector | On/Off 250 Times | Plug and Unplug | Electrically Functional |

Camera  Inspection Standard

| Inspection Item |  |  | Inspection Method | Standard of Inspection |
| --- | --- | --- | --- | --- |
| Category |  | Item |  |  |
| Appearance | FPC/ PCB | Color | The Naked Eye | Major Difference is Not Allowed. |
|  |  | Be Torn/Chopped | The Naked Eye | Copper Crack Exposure is Not Allowed. |
|  |  | Marking | The Naked Eye | Clear, Recognizable (Within 30cm Distance) |
|  | Holder | Scratches | The Naked Eye | The Inside Crack Exposure is Not Allowed |
|  |  | Gap | The Naked Eye | Meet the Height Standard |
|  |  | Screw | The Naked Eye | Make Sure Screws Are Presented (If Any) |
|  |  | Damage | The Naked Eye | The Inside Crack Exposure is Not Allowed |
|  | Lens | Scratch | The Naked Eye | No Effect On Resolution Standard |
|  |  | Contamination | The Naked Eye | No Effect On Resolution Standard |
|  |  | Oil Film | The Naked Eye | No Effect On Resolution Standard |
|  |  | Cover Tape | The Naked Eye | No Issue On Appearance. |
| Function | Image | No Communication | Test Board | Not Allowed |
|  |  | Bright Pixel | Black Board | Not Allowed In the Image Center |
|  |  | Dark Pixel | White board | Not Allowed In the Image Center |
|  |  | Blurry | The Naked Eye | Not Allowed |
|  |  | No Image | The Naked Eye | Not Allowed |
|  |  | Vertical Line | The Naked Eye | Not Allowed |
|  |  | Horizontal Line | The Naked Eye | Not Allowed |
|  |  | Light Leakage | The Naked Eye | Not Allowed |
|  |  | Blinking Image | The Naked Eye | Not Allowed |
|  |  | Bruise | Inspection Jig | Not Allowed |
|  |  | Resolution | Chart | Follows Outgoing Inspection Chart Standard |
|  |  | Color | The Naked Eye | No Issue |
|  |  | Noise | The Naked Eye | Not Allowed |
|  |  | Corner Dark | The Naked Eye | Less Than 100px By 100px |
|  |  | Color Resolution | The Naked Eye | No Issue |
| Dimension |  | Height | The Naked Eye | Follows Approval Data Sheet |
|  |  | Width | The Naked Eye | Follows Approval Data Sheet |
|  |  | Length | The Naked Eye | Follows Approval Data Sheet |
|  |  | Overall | The Naked Eye | Follows Approval Data Sheet |